Ling-TR3L Wireless Serial Port Module

I. Overview

The Ling-TR3L utilizes spread spectrum technology, characterized by development-free, transceiver integration. With the module communication line-of-sight distance of about 2000 meters, users can flexibly configure parameters like power, baud rate, channel and network ID. As long as the understanding of the serial port communication, without the need for complex knowledge of wireless communications, you can complete the development of wireless remote control or data acquisition products.

II. Features

●  Spread spectrum communication, strong anti-interference ability.

●  ISM band, 433Mhz

●  40 adjustable channels

●  Voltage: 2.2-3.6V (typical 3.3V)

●  Receive sensitivity: -130dBm @2.4kbps

●  Transmit current: 68mA

●  Receive current: 9.5mA

●  Sleep current: 6μA

●  UART serial port

●  Operating frequency can be set, multiple modules use frequency division multiplexing technique, mutual non-interference.

●  The RF transceiver switching is automatically completed without user intervention, simple and easy to use.

●  The communication rate ranges from 0.6kbps to 115.2kbps, and can be configured through serial port instructions.

●  The transmission distance is long. In an open area without interference, the line-of-sight distance can reach 2000 meters.

III. Applications

●  Remote Control

●  Data Acquisition

●  Smart Home

●  Industrial Controls

●  Robotics

●  Smart Home Appliances

IV. Pins

| Pin | Name | Function | Description |
| --- | --- | --- | --- |
| 1 | VCC | Power | 2.2–3.6V (typical 3.3V) |
| 2 | RXD | Data Input (TTL level) | UART receive |
| 3 | TXD | Data Output (TTL level) | UART transmit |
| 4 | SET | Set Position | Configuration parameter enable (low to enable parameter configuration, high to suspend) |
| 5 | CS | Sleep | High/suspended: sleep; Low: active |
| 6 | GND | Power Ground | Power Ground |
| 7 | ANT | External antenna interface | 50Ω |

V. Dimensions

VI. Usages

- Communication Mode

- Sleep mode (no data communication, static current about 6uA)

- Setup Mode

VII. Quick Start

After you get the module, you don't need to make any parameter Settings. As long as the circuit connection is correct (refer to Section 6), you can directly carry out serial port communication. You can also use the serial port DEMO below to quickly enter the data transmission and reception test.

- Serial port DEMO (Test board)

- Parameter setting:

- (Non-essential operation, the default parameters can communicate)

- Recommended to use the above tool for setting and testing, if you fail to set the parameters with the upper computer, please check whether the CE pin is low level, whether SET pin is low level, at this time the module light is always on.

- If communication fails, first check the hardware problem. If it is confirmed that there is no problem with the hardware connection, then check whether the parameters of the transceiver module are consistent.

- The setting software can be downloaded from the official website WWW.FNRF.CC.

- When the test communication settings are completed, remember to change the SET back to high level and return to the communication mode.

- When setting parameters with a single-chip microcomputer or a serial port assistant, please write the instructions strictly in accordance with the specifications. The checksum must be correct. Whether the setting is successful can be referred to by querying the instructions.

VIII. Setting instructions

In the normal working mode (i.e., CS low level), the SET pin must be at a low level and the blue LED on the module is constantly on. Then it enters the setting mode, in which the baud rate is forced to be 9600, regardless of the set wireless communication baud rate.

| Parameter | Setting |
| --- | --- |
| 1 | 0xaa (command byte) |
| 2 | 0x5a (command byte) |
| 3,4 | Product ID 2Byte (product model ID, read only, setting is invalid) |
| 5,6 | Net ID 2Byte (network ID, read/write, software grouping, must be the same to communicate, very important) |
| 7 | Nc(reserved) |
| 8 | RF Power 1Byte, setting transmit power, read/write, factory default is max. |
| 9 | Bandwidth (represented by 1-3, corresponding to 125/250/500KHZ) |
| 10 | Baud 1Byte (0-8), Baud rate (600-115200), read/write (default is 4, i.e. 9600) |
| 11 | Nc (reserved) |
| 12 | RF channel 1Byte (0-40 corresponds to 423.92-443.92MHZ) |
| 13 | SF spreading factor, the larger the number, the slower the transmission, but the longer the distance. |
| 14 | Code rate, with the number 0-3 represents the corresponding different values, please refer to the tool, generally choose the default can be. |
| 15-17 | Nc (reserved) |
| 18 | CheckSum 1Byte (all the above bytes are added together) to keep the lower 8 bits. |

Attention:

- "Product ID" is the manufacturer identification code of the product model, read-only, set invalid.

- If you want multiple modules do not interfere with each other, you can set up different NET ID and channels, the former is equivalent to the software grouping, the latter is equivalent to the frequency grouping on the hardware, the combination of hardware and software to use better results.

- For the first use, you can directly use the default parameters for testing. If you need to set parameters using the above table protocol, it is recommended to first use and test with our company's accompanying setting tool. The setting tool has a data monitoring window for sending and receiving commands to observe data changes, which will be very convenient to transplant to the MCU.

- Example:

- Send down parameters to the module (must be in hexadecimal format, as follows)

- AA 5A 00 00 12 34 00 00 02 04 00 14 02 00 00 00 00 66

When the module receives the data, the blue LED flashes and responds with the following data

AA 5B 4C 04 12 34 00 00 02 04 00 14 02 00 00 00 00 B7

The following is the specific format description that is sent down to the module:

AA 5A 00 00 12 34 00 00 02 04 00 14 02 00 00 00 00 66

(The shaded parts for the key fields, other parts for the format to fill in. The shaded parts are parsed in sequence as follows. The non-shaded parts only occupy positions and are not parsed.)

AA 5A : Frame header

12 34 ：Network ID

00 : Transmit power, which can be set from 0 to 16.

02 : Bandwidth, optional 1-3, corresponding to 125, 250, 500KHZ respectively.

04 : Baud rate, corresponding to 9600, with 600/1200/2400/4800/9600/19200/38400/57600/115200 optional.

14 : Channel, corresponding to 20 in decimal, that is, the channel is 433.92MHZ, optional 0-40 corresponds to 4239.2-443.92MHZ.

02 : Spreading factor, optional 0-4, corresponding to SF5-SF9 respectively.

00 : Code rate, generally set to 0 by default, optional 0-3, corresponding to 4/5, 4/6, 4/7, 4/8 respectively.

66 : The sum of the first 17 bytes of the data in this package, take the lower 8 bits.

After the module receives and responds to the data, the frame header changes from AA5A to AA5B, and the Product ID becomes the default number of the manufacturer. Other data is copied to the original command, with the same format as the sending command. The specific analysis is omitted.

Query parameter command :  AA 5C 00 00 12 34 00 00 02 04 00 14 04 00 00 00 00 6A (HEX format)

The module response format is the same as the response format when AA5A was issued, except that the frame header is changed to AA5D.

Query version number command : AA 59 00 00 12 34 00 00 02 04 00 14 04 00 00 00 00 67 (HEX format)

Module response : Return ASC2 code format (including some Chinese) string

Factory default parameter setting : AA 58 4C 04 00 00 00 00 01 04 00 14 02 00 00 00 00 6F (HEX format)

Operating Instruction

After power-on, when the CS pin is connected to low power, the module will work. The SET pin is the setting flag bit. Pulling it low or short-circuiting it to ground can switch the module to the parameter configuration mode.

Setup Mode

Once entering the setup mode, the indicator light will be on. The SET pin configuration must be low, the CS pin must be connected to low, and the serial port Settings will automatically recover to: data bit 8, baud rate 9600, check bit N, stop bit 1.

Indicator light description:

Indicator light is always on in the setup state, off in the working state, received data and transmit data indicator light will blink.

Time Parameters:

| Time Parameter | Typical value |
| --- | --- |
| Power-on initialization time | 160ms |
| CS pull high or suspend to enter sleep | 5.1ms |
| CS pull low to exit sleep time | 49.3ms |
| SET pull low to enter setup time | 10ms |
| SET exit setup time | 54ms |

IX. Antennas

Antennas are critical. Poor or missing antennas severely degrade performance. Actual performance is also related to the PCB, shell, structure, etc. It is recommended to buy more antennas to facilitate the actual test:

Scan the wechat code to purchase antennas

X. Schematic Symbols and Package

Recommended PCB design tool: LCEDA (www.leeda.cn). Search for "FNRF" or "model number" can be found!

Scan the wechat code for technical consultation and to obtain development materials

Scan the wechat code to purchase and get a free tool