Wireless module Ling-TR2 specification

1、Introduction

Ling-TR2 adopts 2.4G SOC technology, which is development-free; module communication is about 100 m, serial transparent transmission and high integration of the serial communication can complete the development of remote control products and data transmission products.

2 、Characteristics

● semi-duplex communication

● 2.4GHz ISM band

● power supply voltage 2.8-3.6V

● reception sensitivity -95dBm

● emission operating current 40mA

● receives an operating current 27mA

● sleep current 9.5uA

● standard TTL level UART serial port

● frequency can be set, with multiple modules without interference

● sending and receiving switching is automatically completed,

● communication rate is 0.6kbps-38.4kbps

3、Pin definition

| pin | Lead the foot name | Feet function | description |
| --- | --- | --- | --- |
| 1 | VCC | + | 2.8~3.6V, Typical 3.3V |
| 2 | RXD | Module data input (TTL level) | Serial port communication data reception |
| 3 | TXD | Module data output (TTL level) | Serial port communication data transmission |
| 4 | SET | Set position | Configuration parameters enable (low level is configuration mode, suspended or high level is communication mode) |
| 5 | CS | dormancy | Pin working at low level, suspended or dormant at high level |
| 6 | GND | - | - |
| 7 | ANT | External antenna interface | When connecting with the external antenna, the resistance at the PCB antenna connection should be removed. |

4、Product size